Data Collection Report: Google Earth Engine (Sentinel-2 Imagery) for Solar Assessment
This report summarizes the process of retrieving satellite imagery using Google Earth Engine (GEE) to support large-scale rooftop solar assessment under the Eco Innovators Hackathon initiative. The aim of this phase was to obtain consistent, cloud-free, and temporally recent imagery suitable for classification and analysis.

Data Source Overview
Satellite imagery was collected using Google Earth Engine (GEE) with the Sentinel-2 Surface Reflectance (COPERNICUS/S2_SR) dataset. Sentinel-2 provides 10 m multispectral imagery with a 5–10 day revisit rate, making it useful for large-scale solar assessment. Unlike high-resolution sources (Google Maps, ESRI, Mapbox), Sentinel-2 cannot show individual rooftop panels. However, it offers consistent, cloud-filtered, and up-to-date imagery. This makes it ideal for research, analytics, and hackathon-level solar studies.

Data Collection Methodology
A structured workflow was followed to maintain accuracy and reproducibility:
1. Dataset Preparation: A list of coordinates (sample ID, latitude, longitude, classification label) was prepared and validated for geospatial accuracy.
2. Temporal Filtering: Imagery was restricted to January–December 2024 to ensure recent and temporally consistent data.
3. Cloud Filtering: For each coordinate, GEE automatically selected the least cloudy Sentinel-2 image, removing scenes with atmospheric noise.
4. Region Creation: Each point was buffered by 1 km, generating a 2 km × 2 km tile for contextual coverage of the surrounding area.
5. Imagery Extraction : bands (B4, B3, B2) were selected, reflectance was normalized, and each tile was exported as a 512 × 512 pixel image named {sampleID}S2{label}.png.
All images were saved in a structured directory for easy organization.
6. Metadata Logging: For every downloaded tile, logs recorded, Coordinates, Selected date, Cloud score, File name and status. This ensures transparency and reproducibility.
7. Quality Control: Images were reviewed for clarity, cloud obstruction, and location accuracy. Poor-quality tiles were flagged or excluded.
8. Failure Handling: Retrieval issues such as high cloud cover or missing data were logged, and problematic tiles were skipped gracefully.

Retrieval Summary
Approximately 90–95% of coordinates yielded usable imagery, while 5–10% were unusable due to persistent clouds or missing data. Sentinel-2 provided strong temporal coverage but lower spatial detail.

India-Specific Considerations
Sentinel-2’s 10-meter resolution limits rooftop-level visibility in India; individual solar panels cannot be identified. However, its frequent revisit rate, free access, and nationwide availability make it valuable for regional solar potential mapping and environmental analysis.

Conclusion
Google Earth Engine provided a fast, consistent, and scalable method for retrieving recent, cloud-filtered Sentinel-2 imagery. Though not suitable for detecting individual rooftop solar panels, GEE’s automation, free access, and multispectral data make it ideal for large-area solar assessment, training datasets, and research-focused modeling.